LAB REPORT

IT3280E– 152049– Assembly Language and Computer Architecture Lab

Lab 02: Instruction Set, Basic Instructions, Compiler Directives

# Assignment 1: Assign 12-bit integer numbers/small integer:

- Instruction <lb>, <sb>:

+ lb is used to load a byte from memory into a register.

+ sb is used to store a byte from a register to memory.

- Instruction Formats:

+ addi: I-type format.

+ add: R-type format.

- When we modify the instruction addi as below:

will gives the result of:

# Assignment 2: Assign 32-bit integer:

- Monitor Register Values:

+ lui s0, 0x20232: This instruction loads the upper 20 bits of 0x20232 into the s0 register, making s0 = 0x20232000.

+ addi s0, s0, 0x024: adds 0x24 to s0, resulting in s0 = 0x20232024.

- Instruction Formats:

+ lui: U-type instruction.

+ addi: I-type instruction.

- Compare data in Data Segment and machines code in Text Segment:

- The data in the Data Segment matches the machine code from the Text Segment, confirming that the binary representations of the instructions are correctly loaded into memory.

# Assignment 3: New assignment instructions:

- Compare:

+ <Source> column: you get the exact same code as you typed.

+ <Basic> column: actual instruction that RARS translates into. (“li” is a pseudoinstruction)

- Explain:

+ li s0, 0x20232024:

- This large immediate value will be split into multiple instructions.

- Likely, it will be broken down into a lui instruction and an addi (Add Immediate) instruction:

  - lui s0, 0x20232 to load the upper part of the value.

  - addi s0, s0, 0x24 to add the lower part (24).

- These instructions together load the full 0x20232024 into s0.

+ li s0, 0x20:

- Since 0x20 is a small value that fits within 12 bits, the pseudoinstruction li s0, 0x20 will be directly translated into a single addi instruction:

addi s0, zero, 0x20

- For small values, it gets translated to a single addi instruction. For large values, it gets translated into multiple intrusctions.

# Assignment 4: Calculate the expression 2x + y = ?:

- The value of s0 after the final instruction is 9 => which is correct

- Machine codes:

+ 0x00500313: addi

+ 0xfff00393: addi

+ 0x00630433: add

+ 0x00740433: add

- Compile to machine code manually:

+ addi t1, zero, 5:

imm[11:0] | rs1 | funct3 | rd | opcode

000000000101 | 00000 | 000 | 00110 | 0010011

- Machine code: 0x00500313

+ addi t2, zero, -1:

imm[11:0] | rs1 | funct3 | rd | opcode

111111111111 | 00000 | 000 | 00111 | 0010011

- Machine code: 0xFFF00393

+ add s0, t1, t1:

funct7 | rs2 | rs1 | funct3 | rd | opcode

0000000 | 00110 | 00110 | 000 | 01000 | 0110011

- Machine code: 0x00630333

+ add s0, s0, t2:

funct7 | rs2 | rs1 | funct3 | rd | opcode

0000000 | 00111 | 01000 | 000 | 01000 | 0110011

- Machine code: 0x00740333

- Same as those displayed in the <Text Segment> window.

# Assignment 5: Multiplication:

- t1 hold the values of X = 4.

- t2 hold the values of Y = 5.

- s1 store the results of the multiplication (which is 20 or 0x014 in hex code).

- Division instructions:

div s2, t1, t2

- s2 will have a value of 0 because the integer division of 4/5 results in 0.

# Assignment 6: Declare and access variables:

- Compare values:

+ X = 5, Y = -1, Z = 0 => match their intial values in the code

- Instruction la:

+ la loads the address of a label (X, Y, or Z) into a register.

+ It’s a pseudo-instruction, so it gets converted into two machine instructions: a lui and an addi.

- Monitor lw and sw:

+ lw is used to load the values of X and Y from memory into registers t1 and t2.

+ sw stores the result of the calculation (Z) back into memory from register s0.

- Instructions lb and sb:

+ lb loads a single byte of data into a register.

+ sb stores a single byte from a register into memory.

# Assignment 7: Declare variables or instructions at specified addresses: